# SIGINT Radio Intercept & Analysis Lab

## Overview

Signals intelligence (SIGINT) involves the interception and collection of information from electronic signals to understand an adversary’s activities and intentions【338585790782648†L76-L89】. This lab will give you hands‑on experience using a software‑defined radio (SDR) to capture, analyse and classify radio transmissions. You will explore a range of signal types, learn basic decoding techniques and document the results in an intelligence log.

## Prerequisites

- A software‑defined radio device, such as an RTL‑SDR dongle. These inexpensive receivers cover a wide frequency range.

- A computer running Windows, macOS or Linux with one of the following SDR applications installed: GQRX, SDR# (SDRSharp) or GNU Radio.

- Optional decoders for specific protocols, such as dump1090 for ADS‑B aircraft beacons or gr‑satellites for satellite telemetry.

- Knowledge of radio frequency bands and basic signal theory.

## Step‑by‑Step Instructions

### 1 Set Up the SDR

- Connect the SDR device to your computer. If using Windows, install drivers (e.g., using Zadig for RTL‑SDR). On Linux, install the rtl-sdr package.

- Launch your chosen SDR software. Set the sample rate (e.g., 2 Ms/s) and choose your SDR device as the input.

- Calibrate the frequency (ppm correction) if necessary. Some dongles drift slightly from the true frequency.

### 2 Capture and Listen to Simple Signals

- FM Broadcast: Tune to the FM radio band (88–108 MHz). Choose a station and verify that you can hear audio. This confirms that your setup works.

- Weather Broadcasts: Tune to local weather radio frequencies (e.g., 162.400 MHz in North America). Listen to the automated weather announcements.

### 3 Explore Other Bands

- Aircraft ADS‑B: Set the frequency to 1090 MHz and run dump1090 (or a similar decoder). This protocol broadcasts aircraft identification and location data. Visualise decoded messages on a map.

- Maritime AIS: Tune to 161.975 MHz and 162.025 MHz. Use an AIS decoder to see ship positions and messages.

- Amateur Radio: Explore the 144 MHz (2 m) or 433 MHz (70 cm) amateur bands. Listen to voice transmissions, Morse code or digital modes (e.g., APRS at 144.39 MHz in the US).

- Satellite Downlinks: If you have a good antenna, tune to 137 MHz to receive NOAA weather satellite images or to the 2400 MHz band for amateur satellite telemetry.

### 4 Classify and Log Signals

- For each signal you intercept, record the following in a logbook:

  - Date and time (UTC).

  - Frequency and bandwidth.

  - Modulation type (e.g., FM, AM, FSK).

  - Content (e.g., music, voice, data, encrypted). If you decode data, capture the message.

  - Category (e.g., COMINT for communications, ELINT for non‑communications electronics). COMINT focuses on intercepting and analysing communications between people【338585790782648†L140-L151】, while ELINT deals with non‑communication emissions such as radar signals【338585790782648†L154-L160】.

- Optionally, use direction‑finding techniques or two SDRs at different locations to estimate the transmitter’s location. Document your methods and accuracy.

### 5 Decode Digital Protocols

- Install decoders specific to the signals you want to analyse. For example, gr-satellites or meteor_demod for weather satellites, rtl_433 for 433 MHz sensors and IoT devices, or multimon-ng for APRS.

- Use these tools to demodulate the raw IQ data and extract human‑readable messages.

- Compare decoded data against open databases (e.g., aircraft registration logs) to add context to your intercept log.

### 6 Ethics and Legal Considerations

- Many countries allow reception of unencrypted transmissions, but intercepting or decoding encrypted communications may be illegal. Know your local laws before tuning into sensitive frequencies.

- Use your SDR responsibly and respect the privacy of others. Do not publish sensitive content you intercept.

### 7 Document Your Findings

- Summarise the types of signals you captured, categorising them as COMINT or ELINT. Note any particularly interesting transmissions.

- Discuss the utility of SIGINT for understanding adversary capabilities and intentions【338585790782648†L76-L89】.

- Include screenshots of your SDR waterfall display and graphs of decoded data where appropriate.

## Next Steps

- Experiment with more advanced SDR tools such as GNU Radio to build custom demodulators and filters.

- Participate in amateur radio and satellite listening communities to expand your knowledge of signal types.

- Combine radio interception with open‑source data (e.g., flight tracking websites) to enrich your analysis.